COMPUTER NETWORKING

MID TERM (ESSAY)

KHEN MUHAMMAD CAHYO

244107023002

TI-2I

STATE POLYTECHNIC OF MALANG

INFORMATION TECHNOLOGY DEPARTMENT

ANSWER QUESTION A

| Department | Network Address | Subnet Mask | Usable IP Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| R&D | 172.16.14.0/2 | /28 | 172.16.14.1 - 172.16.14.14 | 172.16.14.15 |
| Logistics | 172.16.14.16/27 | /27 | 172.16.14.17 - 172.16.14.30 | 172.16.14.31 |
| CS | 172.16.14.32/29 | /29 | 172.16.14.33 - 172.16.14.38 | 172.16.14.39 |
| Data Center | 172.16.14.40/26 | /26 | 172.16.14.41 - 172.16.14.62 | 172.16.14.63 |

ANSWER QUESTION B

The Logistics Director's PC uses the last available host IP in their subnet, which is the last usable IP in the 172.16.14.16/27 subnet.

- Usable IP Range for Logistics: 172.16.14.17 - 172.16.14.30

- The last usable IP is 172.16.14.30.

Thus, the Logistics Director's PC IP Address is 172.16.14.30.

ANSWER QUESTION C

The Data Center requires 60 hosts. The nearest power of 2 that satisfies the requirement is 64 IP addresses, which corresponds to a /26 subnet mask.

Here's the breakdown of the /26 subnet:

- Subnet Mask: /26 → This means the first 26 bits are used for the network portion, leaving 6 bits for the host portion.

- Total Number of IP Addresses: 2 ^ 6 = 64 IP addresses.

  - Out of these 64 IP addresses:

    - 1 address is reserved for the network address.

    - 1 address is reserved for the broadcast address.

    - The remaining 62 addresses are available for hosts.

By using a /26 subnet mask, we ensure that the Data Center has more than enough IP addresses (62 usable IP addresses) to accommodate the required 60 hosts.